Outreach Project - Delivering Medicines to Neonates

Activity Help Sheet on Manufacturing Methods for Making a Tablet/Pill

This help sheet will contain useful information on the type of manufacturing methods which are used to make tablets or pills. You can use this to decide which manufacturing method you think would be best for making your medicine.

Key definitions:

Aggregate – to piece several things together (Cambridge Dictionary, 2022)

API – this means Active Pharmaceutical Ingredient. This is the principal component of the medicine that allows the medicine to impact the consumer in the desired way e.g. in cough syrup, the API is dextromethorphan which is a cough suppressant (VeryWell Health, 2023).

Binder – an adhesive chemical usually sprayed on particles during wet granulation so they can bond together (Anon, n.d.)

Granulation – a process that is used in the pharmaceutical industry to form grains or granules from a powdery or solid substance (Cambridge Dictionary, 2023)

Dry granulation (roller compaction)

This is when you combine your granules but without liquid; this happens if your mixture is sensitive to moisture and heat. The granular mixture is fed through a roller compactor where two rollers will compact the mixture to form a ribbon, which will then travel through a granulator, where the granules are forced through a mesh, so they come out at the desired size (Hsu, Reklaitis and Venkatasubramanian, 2010).

This diagram below shows roller compaction (Hsu, Reklaitis and Venkatasubramanian, 2010).

Advantages:

- There is no moisture or heat introduced to the mixture (Mittal, 2016)

- The process is continuous (Mittal, 2016)

Disadvantages:

- If the drug substance is not properly compressed, then it may stick on the rollers (Mittal, 2016)

Wet granulation (high shear)

This is when you use a liquid solution and add it to your granules so the particles can bond together. To aggregate the particles, you can use a non-toxic binder and spray it onto the formulation so the particles bond. The liquid binder is added at a lower impeller and chopper speed. Wet massing, which is when the dry powder particles mix with the granulating fluid, occurs when both the impeller and chopper are at high speeds (Gokhale and Trivedi, 2016).

The diagram below shows a high shear machine (Liu et al, 2021).

Advantages:

- The scale-up process is easy (Mittal, 2016)

- A granulation bowl can be used for blending the mixture (Mittal, 2016)

Disadvantages:

- It is a two-step process – granulation and then drying, which may make this time-consuming (Mittal, 2016)

Wet granulation (fluidised bed)

This is the same process done as above, but in a fluidised bed granulator. Hot air is fed into the bed which lifts the granules, whilst they are sprayed the granules aggregate together to form bridges. This is the first stage. In the second stage, the powder is wetted to form liquid bridges. The process is continued till the required size of granule is formed with solid bridges between the granules. Once the fluidised bed granulator is turned off, the hot air will continue to flow through the granulator and so the granules will dry (Gupta, 2017).

This is a diagram of a fluidised bed granulator (Gupta, 2017).

Advantages:

- It is simple as it only needs one pot for processing (Mittal, 2016)

- There is minimal amount of moisture during granulation (Mittal, 2016)

- There is general homogeneity in the granules produced – they flow free and have a lower density compared to granules formed by other granulation methods (Mittal, 2016)

Disadvantages:

- Since both moisture and heat are introduced at the same time, there are many process variables that need to be considered for example, thermodynamic properties (Mittal, 2016)

References:

Anon, (n.d.). Binding Agents - Shellac, Waxes, Natural Gums, Resins, Menthol Crystals & Zein. [online] Available at: https://www.afsuter.com/product-category/application/binding-agents/#:~:text=Liquid%20binders%20are%20added%20to [Accessed 26 Jul. 2023].

Cambridge Dictionary (2022). aggregate. [online] @CambridgeWords. Available at: https://dictionary.cambridge.org/dictionary/english/aggregate.

Cambridge Dictionary (2023). granulation. [online] @CambridgeWords. Available at: https://dictionary.cambridge.org/dictionary/english/granulation [Accessed 26 Jul. 2023].

Gokhale, R. and Trivedi, N.R. (2016) Wet granulation in low-and high-shear mixers. In Handbook of pharmaceutical granulation technology (pp. 199-219). CRC Press.

Gupta, R. (2017) Fluid bed granulation and drying. In Predictive Modeling of Pharmaceutical Unit Operations (pp. 137-158). Woodhead Publishing.

Gupta, R. (2017) Schematic of a typical top spray fluid bed unit. [Online]. Available at: https://www.sciencedirect.com/science/article/pii/B9780081001547000065 (Accessed: 26/07/2023).

Hsu, SH., Reklaitis, G.V. & Venkatasubramanian, V. (2010) Dry Granulation [Online]. Available at: https://link.springer.com/article/10.1007/s12247-010-9076-0#citeas (Accessed: 26/07/2023).

Hsu, S.H., Reklaitis, G.V. and Venkatasubramanian, V. (2010). Modeling and control of roller compaction for pharmaceutical manufacturing. Part I: Process dynamics and control framework. Journal of Pharmaceutical Innovation, 5, pp.14-23.

Liu, B., Wang, J., Zeng, J., Zhao, L., Wang, Y., Feng, Y. and Du, R. (2021) Schematic diagram of the granulating module of the commonly used high shear wet granulator. [Online]. Available at: https://www.sciencedirect.com/science/article/pii/S0032591020311104 (Accessed: 26/07/2023)

Mittal, B. (2016) How to develop Robust solid oral dosage forms: from conception to post-approval. Academic press.

Verywell Health. (2023). What Is an Active Pharmaceutical Ingredient in Medication? [online] Available at: https://www.verywellhealth.com/api-active-pharmaceutical-ingredient-2663020.